SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1 – ANALYTICAL PROBLEM SOLVING (CSA1209 – Computer Architecture)

ANNEXURE A – Worked Solutions (CO2)

NAME: R.ASIF/192411163

# Question 1 — Signed 8-bit Addition & Subtraction (2's Complement)

Numbers: A = +45, B = −23 (8-bit, 2's complement representation).

## Step 1: Binary conversion

- +45 in 8-bit binary = 0 0101101

- 23 in unsigned binary = 00010111

- −23 in 2's complement = invert bits of 00010111 (→ 11101000) and add 1 → 11101001

## Step 2: Addition — A + B = 45 + (−23)

Align both operands as 8-bit words and add, tracking the carry out of every bit position:

| Bit position | 7 (MSB) | 6 | 5 | 4 | 3 | 2 | 1 | 0 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +45 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| −23 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| Carry generated | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| Sum | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |

Result bits = 00010110₂ = +22 (decimal). A carry of 1 is generated out of the MSB (bit 7); this carry is simply discarded in signed addition (it is not an error by itself). Check: 45 + (−23) = 22 ✔

## Step 3: Subtraction — A − B = 45 − (−23)

Subtraction is performed as addition of the 2's complement of the subtrahend: A − B = A + (−B). Since B = −23, −B = +23 = 00010111.

| Bit position | 7 (MSB) | 6 | 5 | 4 | 3 | 2 | 1 | 0 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +45 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| +23 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| Carry generated | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Sum | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |

Result bits = 01000100₂ = +68 (decimal). Check: 45 − (−23) = 45 + 23 = 68 ✔

## Step 4: Overflow detection

A processor detects signed overflow by comparing the carry into the sign bit (Cin at bit 7) with the carry out of the sign bit (Cout at bit 7):

Overflow = Cin(MSB) XOR Cout(MSB)

| Operation | Carry into bit 7 (Cin) | Carry out of bit 7 (Cout) | Overflow = Cin Cout | Interpretation |
| --- | --- | --- | --- | --- |
| Addition (45 + (−23)) | 1 | 1 | 0 → No overflow | Result 00010110 = +22, correct and in range [−128,127] |
| Subtraction (45 − (−23)) | 0 | 0 | 0 → No overflow | Result 01000100 = +68, correct and in range [−128,127] |

An alternative, equivalent rule the ALU hardware also uses: overflow occurs only when both operands have the same sign but the result has a different sign. In both operations above this does not happen, so no overflow flag is set and both results (+22 and +68) are correctly represented in 8-bit signed form.

# Question 2 — 4-bit Carry Look-Ahead Adder (CLA)

Consider two 4-bit binary inputs A = 1011 (11) and B = 0110 (6), with Cin = 0, added using a 4-bit CLA.

## Step 1: Generate (G) and Propagate (P) signals

For each bit position i: Gᵢ = Aᵢ · Bᵢ (a carry is generated at this stage regardless of Cin), and Pᵢ = Aᵢ  Bᵢ (an incoming carry is propagated through this stage).

| i | Aᵢ | Bᵢ | Gᵢ = Aᵢ·Bᵢ | Pᵢ = Aᵢ Bᵢ |
| --- | --- | --- | --- | --- |
| 0 (LSB) | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 |
| 3 (MSB) | 1 | 0 | 0 | 1 |

## Step 2: Carries computed in advance (look-ahead equations)

C1 = G0 + P0·C0

C2 = G1 + P1·G0 + P1·P0·C0

C3 = G2 + P2·G1 + P2·P1·G0 + P2·P1·P0·C0

C4 (Cout) = G3 + P3·G2 + P3·P2·G1 + P3·P2·P1·G0 + P3·P2·P1·P0·C0

Substituting C0 = 0:

- C1 = 0 + (1)(0) = 0

- C2 = 1 + (0)(0) + (0)(1)(0) = 1

- C3 = 0 + (1)(1) + (1)(0)(0) + (1)(0)(1)(0) = 1

- C4 = 0 + (1)(0) + (1)(1)(1) + (1)(1)(0)(0) + (1)(1)(0)(1)(0) = 1

Every carry (C1, C2, C3, C4) is generated directly from A, B and C0 through two levels of gates — none of them waits for the carry chain of a ripple adder.

## Step 3: Sum bits and final result

Sᵢ = Pᵢ  Cᵢ

| i | Pᵢ | Cᵢ | Sᵢ = Pᵢ Cᵢ |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| i | Pᵢ | Cᵢ | Sᵢ = Pᵢ Cᵢ |
| 1 | 0 | 0 | 0 |
| 2 | 1 | 1 | 0 |
| 3 | 1 | 1 | 0 |

Sum = S3S2S1S0 = 0001, Cout = C4 = 1 → combined result = 1 0001₂ = 17 decimal. Check: 11 + 6 = 17 ✔

## Step 4: Delay comparison — CLA vs Ripple Carry Adder (RCA)

| Aspect | Ripple Carry Adder | 4-bit Carry Look-Ahead Adder |
| --- | --- | --- |
| Carry path | Each stage must wait for Cin from the previous stage | All carries computed directly from A, B, C0 in parallel |
| Delay (n-bit) | ≈ n × (2-gate delay per full adder) → grows linearly with n | ≈ constant (2–3 gate levels) regardless of n, plus O(1) extra for the look-ahead logic |
| For n = 4 | ≈ 8 gate delays (worst case, carry ripples through all 4 stages) | ≈ 3–4 gate delays (G/P generation, then carry logic, then sum XOR) |
| Hardware cost | Low — simple, repeated fulladder cells | Higher — extra AND/OR gates for the look-ahead network |

Justification: In an RCA, stage i cannot compute its sum until Ci arrives from stage i−1, so worst-case delay grows linearly with word length (O(n)). A CLA instead pre-computes every Gᵢ and Pᵢ in one gate delay, and every carry Cᵢ is derived directly from C0 and the G/P signals in a second gate delay — independent of n. This is why modern processors, which need to add wide words (32/64 bit) at GHz clock rates, use look-ahead (or hybrid block-CLA) adders: the reduced, near-constant critical path lets the ALU meet a much shorter clock period than a pure ripple design would allow, at the cost of extra gates.

# Question 3 — Booth's Algorithm: (−6) × (+5)

Represent both numbers in 4-bit 2's complement: Multiplicand M = −6 = 1010, Multiplier Q = +5 = 0101. Also compute −M = 0110. Registers: Accumulator A (4 bits, initially 0000), Q (multiplier), Q₋₁ (extra bit, initially 0).

## Booth's rule

- Examine the pair (Q0, Q₋1) at each step.

- 10 → this marks the start of a run of 1's → A = A − M

- 01 → this marks the end of a run of 1's → A = A + M

- 00 or 11 → inside a run of 0's or 1's → no arithmetic operation

- After the add/subtract (or no-op), arithmetic-shift-right the combined register (A, Q, Q₋1) by 1 bit.

## Step-by-step execution (4 cycles, since the multiplier is 4 bits)

| Cycle | Q0 Q₋1 | Operation | A (after op) | A Q Q₋1 (after arithmetic shift right) |
| --- | --- | --- | --- | --- |
| Initial | 1 0 | — | 0000 | A=0000 Q=0101 Q₋1=0 |
| 1 | 1 0 | A = A − M = 0000 − 1010 | 0110 | A=0011 Q=0010 Q₋1=1 |
| 2 | 0 1 | A = A + M = 0011 + 1010 | 1101 | A=1110 Q=1001 Q₋1=0 |
| Cycle | Q0 Q₋1 | Operation | A (after op) | A Q Q₋1 (after arithmetic shift right) |
| 3 | 1 0 | A = A − M = 1110 − 1010 | 0100 | A=0010 Q=0100 Q₋1=1 |
| 4 | 0 1 | A = A + M = 0010 + 1010 | 1100 | A=1110 Q=0010 Q₋1=0 |

## Final result

After 4 cycles: Accumulator A = 1110, Q = 0010. The 8-bit product = A concatenated with Q = 1110 0010₂.

Decoding as an 8-bit 2's complement number: invert (0001 1101) and add 1 → 0001 1110 = 30, so the value is −30.

Check: (−6) × (+5) = −30 ✔

## Why Booth's algorithm is efficient for signed numbers

- It handles positive and negative multipliers uniformly in 2's complement, with no separate signcorrection step at the end (unlike naive add-and-shift multiplication of signed numbers).

- A run of consecutive 1's in the multiplier (e.g. 0111110) is replaced by just one subtraction at the start of the run and one addition at the end, instead of one addition per 1-bit — this can markedly cut the number of add/subtract operations for multipliers with long runs of 1's.

- Every cycle still performs an arithmetic shift (cheap, single gate-delay per bit) regardless of the bit pattern, so the algorithm's cost is dominated by the (at most one) add/subtract per cycle rather than by every individual bit of the multiplier.

# Question 4 — Division: Restoring vs Non-Restoring (13 ÷ 3)

Dividend = 13 = 1101₂, Divisor = 3 = 0011₂, using n = 4-bit operands and a 5-bit remainder register A (initially 0000 0) to accommodate the sign during trial subtraction. M = 00011, −M (2's complement) = 11101.

## (a) Restoring Division

Rule for each of the n = 4 cycles: (1) shift the (A, Q) pair left by 1 bit; (2) A = A − M; (3) if A is negative, set Q0 = 0 and restore A by adding M back (A = A + M); otherwise set Q0 = 1 and keep the subtracted A.

| Cycle | Shift left A,Q | A − M | A negative? | Action | A after cycle | Q after cycle |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | A=00001, Q=1010 | 00001−00011=11110 | Yes | Q0=0, restore A=A+M | 00001 | 1010 |
| 2 | A=00011, Q=0100 | 00011−00011=00000 | No | Q0=1, keep A | 00000 | 0101 |
| 3 | A=00000, Q=1010 | 00000−00011=11101 | Yes | Q0=0, restore A=A+M | 00000 | 1010 |
| 4 | A=00001, Q=0100 | 00001−00011=11110 | Yes | Q0=0, restore A=A+M | 00001 | 0100 |

Final: Remainder = A = 00001 = 1, Quotient = Q = 0100 = 4. Check: 13 = 3×4 + 1 ✔

## (b) Non-Restoring Division

Rule: shift (A, Q) left by 1 bit first. Then, if the previous A was non-negative, A = A − M; if the previous A was negative, A = A + M. Set Q0 = 1 if the new A is non-negative, else Q0 = 0 (no explicit restore during the loop). If the final A is negative, a single corrective addition (A = A + M) is performed at the very end.

| Cycle | Shift A,Q | Op based on previous A sign | A after op | New A sign | Q0 set | Q after cycle |
| --- | --- | --- | --- | --- | --- | --- |
| Cycle | Shift A,Q | Op based on previous A sign | A after op | New A sign | Q0 set | Q after cycle |
| 1 | A=00001,Q=1010 | prev A≥0 → A−M | 00001−00011=11110 | Negative | 0 | 1010 |
| 2 | A=11101,Q=0100 | prev A<0 → A+M | 11101+00011=00000 | Non-neg | 1 | 0101 |
| 3 | A=00000,Q=1010 | prev A≥0 → A−M | 00000−00011=11101 | Negative | 0 | 1010 |
| 4 | A=11011,Q=0100 | prev A<0 → A+M | 11011+00011=11110 | Negative | 0 | 0100 |

Final A (1110₀) is negative → one corrective restore: A = A + M = 11110 + 00011 = 00001. Final: Remainder = A = 00001 = 1, Quotient = Q = 0100 = 4. Check: 13 = 3×4 + 1 ✔ — matching the restoring-division result, as expected.

## Comparison: Restoring vs Non-Restoring

| Criterion | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Operations per cycle | Up to 2 (a subtraction, and — whenever the result is negative — an extra restoring addition) | Exactly 1 (either an addition or a subtraction, decided by the previous sign) — no restore inside the loop |
| Total add/sub ops (this example) | 7 (cycles 1,3,4 needed subtract+restore = 2 each; cycle 2 needed 1) | 5 (1 op per cycle × 4, plus 1 final corrective addition) |
| Control complexity | Simpler condition, but variable/extra operations increase average latency | Slightly more complex control (must remember previous sign), but uniform, predictable 1-opper-cycle timing |
| Hardware/speed | Slower on average due to the extra restoring step | Faster and more regular — better suited to pipelined/hardware implementation |

Non-restoring division is generally preferred in modern processors because it performs exactly one addition or subtraction per cycle (never two), giving a fixed, predictable cycle time that is easier to pipeline in hardware, whereas restoring division's extra conditional 'restore' step adds variable overhead and extra energy/time on every cycle where the trial subtraction fails.

# Question 5 — IEEE 754 Representation and Addition (−13.25)

## Step 1: Convert −13.25 to binary and normalize

- 13 (integer part) = 1101₂

- 0.25 (fractional part) = 01₂ (0.25 = 1×2⁻²)

- So 13.25 = 1101.01₂

- Normalized (scientific) form: 1101.01 = 1.10101 × 2³

- Sign bit = 1 (number is negative)

## Step 2: Single precision (32-bit: 1 sign, 8 exponent, 23 mantissa)

- Exponent field uses bias 127: stored exponent = actual exponent + bias = 3 + 127 = 130 = 10000010₂

- Mantissa (fraction after the implicit leading 1): 10101, padded with zeros to 23 bits → 10101000000000000000000

| Sign (1 bit) | Exponent (8 bits) — bias 127 | Mantissa (23 bits) |
| --- | --- | --- |
| Sign (1 bit) | Exponent (8 bits) — bias 127 | Mantissa (23 bits) |
| 1 | 10000010 | 10101000000000000000000 |

## Step 3: Double precision (64-bit: 1 sign, 11 exponent, 52 mantissa)

● 	Exponent field uses bias 1023: stored exponent = 3 + 1023 = 1026 = 10000000010₂ ● 	Mantissa: 10101, padded with zeros to 52 bits

| Sign (1 bit) | Exponent (11 bits) — bias 1023 | Mantissa (52 bits) |
| --- | --- | --- |
| 1 | 10000000010 | 1010100000000000000000000000000000000000000000000 |

## Step 4: Floating-point addition example (−13.25) + (+6.5)

To illustrate addition, add −13.25 to +6.5 (single precision, for brevity).

- −13.25 = −1.10101 × 2³  (sign=1, exponent=3)

- +6.5 = 110.1₂ = 1.101 × 2²  (sign=0, exponent=2)

## (i) Equalize exponents (align to the larger exponent, 3)

The smaller-exponent operand's mantissa is shifted right by the exponent difference (3 − 2 = 1), and its exponent is raised to match:

6.5:  1.101 × 2²  →  0.1101 × 2³

## (ii) Add/subtract the aligned mantissas

Because the operands have opposite signs, the magnitudes are effectively subtracted (larger magnitude minus smaller), and the sign of the larger-magnitude operand (−13.25) is taken for the result:

1.10101   (13.25's mantissa)

− 0.11010   (6.5's aligned mantissa)

-----------

0.11011   × 2³   (unnormalized result, negative)

## (iii) Normalize

The leading bit is 0, so the result is not normalized. Shift the mantissa left by 1 bit and decrement the exponent by 1 to restore the form 1.xxxx × 2ⁿ:

0.11011 × 2³  →  1.1011 × 2²

So the result is −1.1011 × 2², i.e. −(1 + 0.5 + 0.0625 + 0.03125) × 4 = −1.6875 × 4 = −6.75. Check: −13.25 + 6.5 = −6.75 ✔

## (iv) Rounding and precision-loss considerations

- Rounding: in this example the exact mantissa 1.1011 fits well within the available 23 (or 52) mantissa bits, so no rounding is needed. In general, after the subtract/normalize step the mantissa may need more bits than the format allows — the FPU then rounds (round-to-nearest-even by default in IEEE 754), which can introduce a small rounding error.

- Normalization shift: whenever operands of nearly equal magnitude and opposite sign are added (as here), the leading bits can cancel, forcing a left-shift during normalization. This shift pulls in zero-bits at the least-significant end, so any information that would have occupied those low bits is lost — this effect

is called catastrophic cancellation, and it is the main source of precision loss in floating-point addition of like-magnitude, opposite-sign numbers.

- Alignment shift: shifting the smaller operand's mantissa right during exponent equalization can also shift bits below the mantissa's least-significant position (into 'guard/round/sticky' bits in real hardware); if not tracked, this too becomes a source of rounding error.